2022 Annual Marsh Report

I.  Status of the Marsh in 2022.

This is the fourth year during which the level of the water in the marsh had been allowed to cycle naturally following cessation of the draining of the marsh prior to which the marsh was completely drained in the summer of each of three consecutive years. Nearly every month from September 2021 through July 2022 was wetter than normal. Measurements of rainfall totaled monthly from September 2021 through July 2022, excluding February 2022, compared to 20-yr monthly averages for those same months at a rain gage station near the marsh averaged 159% of normal. From late fall 2021 to early summer of 2022 water levels in the marsh increased to fully flooded, i.e. the level of water was at or flowing over the top board in the water control device (WCD), beginning in mid-November 2021, a month and a half earlier than the previous year, and continuing to the last full week in May 2022, a month later than the previous spring. The depth of water in the marsh typically decreases over summer and into early fall. As late as September 5, 2022 the water level at the WCD was 5.42ft, a depth of water normally recorded in July.

On July 25, 2022 beaver activity was first observed at the WCD. Apparently, a leak between two of the lower boards of the WCD that was producing the sound of flowing water had attracted the beaver. The animal has been in the process of building a dam consisting of tree branches, aquatic vegetation, mud and some rocks in front of the WCD up to the present.

By September 4, 2022 the beaver had amassed a dam with a surface area of about 50 sq ft and a volume of about 4.5 cu yd. The dam building effort of the beaver has succeeded in staunching the flow of water through the leak. As a result the beaver dam is at present seriously compromising the ability of the WCD to control the level of water in the marsh.

This is not the first time that a beaver has attempted to build a dam at the WCD. In June 2016 Grant Calverley discovered evidence of a beaver beginning to build a dam at the WCD. In that year the Marsh Committee had decided to drain the marsh in preparation for mowing cattails. The evidence of beaver activity caused a pause in the drawdown of water in the marsh from 6/14/16 to 7/20/16 in order to allow the Marsh Committee to come to a decision about whether or not to continue the drawdown. During the pause it was discovered that the beaver had moved on allowing the drawdown to continue. Having successfully plugged the leak in the WCD it remains to be seen if the current beaver will remain in the marsh or move on.

Another mammal of greater interest to Three Meadows for cattail management is the muskrat. Between 2/7/22 and 5/3/22 I observed muskrat(s) in the marsh on five occasions. On one occasion I saw two muskrats swimming together in the marsh. In past years I usually observed muskrat(s) in the marsh on only one or two occasions each year in the marsh. Although encouraging, the number of observations this year probably do not indicate that muskrats are currently having any appreciable impact on cattail stands in the marsh.

In the 2021 Annual Marsh Report we reviewed evidence that species assemblages of marsh- associated birds had essentially returned to those of pre-drawdown. In this report I review counts of species of interest that occupied the marsh during 2021-2022. Trumpeter Swans occupied the marsh about as extensively in late fall to early spring of 2021-22 as they did over a similar period in 2020-21. Based on the observations of Claudia Fullerton, Rita O’Clair and me swans arrived on the marsh a bit later (15 Nov 2021 vs 30 Oct 2020) and left about the same time (15 March 2022 vs 22 Mar 2021) in 2021-22 as in 2020-21. The frequency of occurrence of the swans on the marsh in 2021-22 (82%) was about the same as that in 2020-21 (86%). The frequency of occurrence of one pair of swans (65%) accounted for most of the observation of swans in the marsh in 2021-22. As a result the average number of swans counted when present on the marsh in 2021-22 (2.6) was lower than that in 2020-21 (3.9). The maximum number of swans counted on the marsh in 2021-22 (7) was also lower than in 2020-21 (13).

Between 1 November 2021 and 31 March 2022 migrating and wintering waterfowl ranged up to 73 individuals. Waterfowl numbers comprising between three and seven species exceeded 50 individuals 33% of the time during the period. At least three species of waterfowl, Mallard, Ring-necked Duck, and Hooded Merganser raised between one and three broods of ducklings depending on the species in the marsh in late spring or summer of 2022.  Although I did not observe any Wood Duck broods in the marsh in 2022, judging from the number of immature Wood Ducks in the marsh in late summer it is likely that at least one pair of Wood Ducks successfully bred there in 2022. I observed at least two broods of Pied-billed Grebes in the marsh in 2022. Other marsh birds were counted at levels generally comparable to counts in 2021. As in 2021 Sora were rarely observed in the marsh in 2022. Unlike in 2021 I observed no evidence of  the American Bittern in the marsh in 2022, although at least one other birder, Phil Green, has both seen and heard an American Bittern in the marsh at various times in 2022.

II.  Cattail Grooming Test.

In the 2019 Annual Marsh Report we considered the appropriateness of each of the three general methods for the control of cattails, chemical, biological, and mechanical, in reference to Three Meadows Marsh.  We argued that a mechanical approach would be the most appropriate for our marsh at this time. Therefore in August 2020 I began a test of a method recommended by Beule (1979) for the control of cattails. The method entails cutting the stems of cattails at least 7 cm below the surface of the water in summer when the water level is lowest. This method in which “more than 90% of the Typha reproduction was killed.”was found to be preferable to cutting the stems above water which resulted in “no apparent kill” (Beule, 1979). A number of years ago Craig Staude working with Liz Francis employed the same method on a test patch near the knoll at the eastern shore of the marsh.  They concluded that the method appeared to be successful in limiting cattail reproduction.

I am employing a commercially available device termed an Aquatic Vegetation Groomer to cut the cattail stems at the desired level. The site of our test is the same as that used for the Pig Experiment (see the 2018 Annual Marsh Report ). For more detail on the test site see the 2020 Annual Marsh Report. Examination of the test area this year indicates that the test has shown  promise for the effective control of cattails.  Cattails were cut using this method over an area of about 650 ft2. Much of that area remains cattail free in the summer of 2022.

III. Plans for 2023

I will continue cattail grooming in the fall of 2022as long as I have access to live cattail stems. I expect late fall and winter rains to begin filling the marsh eventually denying me access to the deeper cattails. In 2023 I plan to further evaluate the method of Buele (1979) by expanding the area of cattail grooming beyond the current test area.

IV. Recommendations

Because of the extremely adverse effect of the beaver dam on the functioning of the water control device our main recommendation is that the beaver dam be removed as soon as possible. The $450 in marsh budget should cover at least a start on the removal. If more money is required to support the removal of the dam, we request that the Three Meadows Board of Directors approve the necessary funds to complete the job.

V.  Literature Cited.

Beule, J. D. 1979. Control and management of cattails in southeastern Wisconsin wetlands. Technical Bulletin No. 112. Madison, WI: Department of Natural Resources. 40 p.

Submitted by,

Chuck O’Clair and Craig Staude

Co-chairmen of the Marsh Committee

Date: 15 September 2022